# Payment Terminal Troubleshooting Guide

# Supported Brands and Processors

## Payment Terminal Brands:

  - Pax A35

  - Pax A920

  - Ingenico Lane3600

  - Ingenico Moby 5500

## Processors:

  - Elavon

  - Tsys

  - Titanium

  - Fiserve

  - World Pay

  - Heartland

  - Bank of America

# POS Connection to the Payment Terminal

## How to Connect Your Payment Terminal

- Identify the brand of your payment terminal. Refer to the list of supported brands above.

- For example:

  - Pax A35

  - Ingenico Lane3600

- Follow the steps provided for your specific terminal model and issue its related.

## How to Connect Your Payment Terminal to Local Network

- Power on the terminal.

- Go to Settings.

- Enter password: 9876 or pax9876@@.

- Select Network.

- Choose Wifi or Network—do not enable both, as this causes conflicts.

- For Wifi > Select the SSID > Enter the password

- For Network > Plug the LAN cable to the Red Port of the payment terminal.

- Note the network’s IP address after connection.

## How to Connect Your Payment Terminal to the POS

Pax A35 and PaxA920

- Open RapidRMS.

- Tap Setting at the bottom center of the home screen.

- Enter your PIN.

- Navigate to Gateway and select Pax.

- Choose Pax Configuration from the side menu.

- Enter the previously noted IP address.

- Set PORT to 10009.

- Choose the appropriate communication protocol, either HTTP or HTTPS, ensuring that it aligns with the configuration of the payment terminal.

- Save and test the connection. If it fails, follow the troubleshooting steps below.

  - Note: When using Rapid Connect, an "Unspecified Error" during test connection means the connection was successful.

## Troubleshooting steps to connect Pax Payment Terminal

Pax A35 and PaxA920

To ensure proper connection between your payment terminal and POS system, follow these steps:

- Set the Communication Type to Ethernet.

- Choose Protocol Type as either HTTP Get or HTTPS Get.

## To verify the ECR settings on your payment terminal:

- Access the settings by tapping each corner of the screen—top left, top right, bottom right, and bottom left. The terminal will ask for a password.

- Enter today’s or tomorrow’s date as the password. If neither works, contact your payment processor to obtain it. (For Bank of America customers, try using your MID.)

- Navigate to ECR Settings.

- Enable Ethernet.

- Select the protocol type: HTTP or HTTPS. Ensure this matches the protocol selected in the POS > Pax configuration.

## Basic Troubleshooting Questions

- Determine if any events occurred before the device was disconnected, such as:

- Power outage

- Network changes

- Ensure your POS system and payment terminal are operating on the same network.

- Verify if the payment terminal has received any recent updates.

# Identifying the Issue

- If you are having trouble connecting between the POS and the payment terminal:

- This issue should be routed for checking the connection between the POS and the payment terminal, following steps based on the terminal type.

- If you receive an error message when a card is swiped, tapped, or inserted on the payment terminal:

- These issues should be routed to the payment processing provider for further diagnosis, as they are not related to the POS system.